A global-local fretting analysis methodology and design study for the pressure armour layer of dynamic flexible marine risers

S.M. O’Hallorana, A.D. Connaireb, A.M. Hartec1, S.B. Leend1*

a SEAM Research Centre, Waterford Institute of Technology, Waterford, Ireland

b Wood plc., Galway Technology Park, Parkmore, Galway, Ireland

c Civil Engineering, National University of Ireland, Galway, Ireland

d Mechanical Engineering, National University of Ireland, Galway, Ireland

1 Joint senior authors

* Corresponding author: Dr. Sinéad O’Halloran, sohalloran@wit.ie

# Abstract

In this paper, a global-local fretting design methodology for the pressure armour layer of flexible marine risers is outlined. This includes global dynamic riser analysis, geometrical and analytical sub-models and local nub-groove contact finite element analysis. Furthermore, a fretting test rig is developed and utilised to quantify coefficient of friction and wear coefficient under representative nub-groove loading conditions. The combination of the global-local computational methodology and experimental characterisation of pressure armour wire material allows for the development of running condition fretting maps. This identifies design criteria for critical riser global curvatures that are associated with minimum number of lives to failure. The design methodology presented in this paper is applied to a realistic riser design study, using extreme sea-state loading conditions. In this case study, the predicted pressure armour fretting fatigue lives are found to be in the same range as theplain fatigue lives of the tensile armour layer.

Keywords:  Flexible marine risers, nub-groove contact, crack initiation, fretting wear, fretting fatigue, fretting case study

# Introduction

Fretting is a surface damage mechanism, which occurs in bodies in contact under a normal, clamping load and cyclical tangential loading. Fretting contact conditions can be classified according to slip regime [1], depending on the slip amplitude as follows: (i) gross slip (for limiting friction conditions), (ii) partial slip (below limiting friction) and (iii) stick. The slip regime depends specifically on the applied combination of normal (clamping) load and tangential load or displacement. Glaeser [2] reported that billions of dollars could be saved if wear and fatigue life of engineering components could be improved. Since then improvements have been made in the understanding and analysis of fretting wear and fatigue performance of components, such as spline couplings, for aerospace applications [3,4], gas turbine dovetail joints [5,6], steel wire ropes [7] and other industrial applications. Many researchers have experimentally investigated fretting wear and fretting fatigue, e.g. [8–13]. It has been shown that the rapidly varying stress fields that occur in fretting fatigue loadings cause a contact size effect on the fretting fatigue life [14–17]. Burke and Witz [18] presented an overview of fretting issues in the pressure armour wire of flexible marine risers at an oil and gas industrial conference. Despite this, the problem of fretting fatigue in flexible marine risers has been the subject of limited research.

Flexible marine risers are vital infrastructure in the transfer of offshore oil and gas from well-heads at the seabed to a floating vessel or platform at sea level. The key engineering design of these risers is the large number of layers (see Figure 1) having distinct functions, giving flexibility and strength to withstand substantial sea-state loading conditions. Due to the combination of extreme sea-state global dynamic loading conditions and the local nub-groove geometrical and tribological conditions (see Figure 1), fretting is a design concern for the pressure armour layer. The principal function of this interlocked metallic layer is to contain internal pressure by resisting hoop stresses. In the case of pressure armour layers, the normal load is primarily due to internal and external fluid pressures (hydrocarbon and hydrostatic) and the tangential load (displacement) is due to bending moment, axial load, torque or a combination of all three, due to vessel motions, wave and current effects on the riser deformation. Fretting wear and fatigue can cause local cracking and damage of the pressure armour layer, resulting in reduced service life of the riser. However, the relationship between global riser loading conditions, such as internal and external pipe pressure, vessel motions, wave and current loads, and local nub-groove fretting conditions, such as contact pressure and contact slip, is unknown. Perera et al. [19] presented a novel experimental setup for the fretting testing of the nub-groove contact of the pressure armour layer of unbonded flexible pipes under representative loading conditions. Féret and Bournazel [20] examined the behaviour of flexible risers under axisymmetric loading and presented an analytical solution for the stresses, contact pressures and slip between tensile armour wires; these parameters are key to the evaluation of fatigue and wear of the tensile armour wires. However, this approach is not applicable to the pressure armour layer. In this work, a step is taken towards quantifying this relationship by developing a global-local fretting methodology for risers and applying it to realistic riser configurations and sea state loading conditions.

Previous work by the authors has experimentally characterised the coefficient of friction and wear coefficient of pressure armour material [21]; it has also been shown computationally that the coefficient of friction can affect fretting crack initiation in risers due to trailing-edge tensile stresses in the nub-groove contact of the pressure armour layer [22]. Furthermore, a wear-fatigue model was developed in which an adaptive meshing technique was used to characterise the influence of nub-groove displacement on fretting fatigue life of the pressure armour layer [23]. It was shown that nub-groove contact pressure and bending-induced axial displacement significantly affect predicted crack initiation. It has also been shown that a surface damage parameter, developed by Ding et al. [24], combined with a multiaxial fatigue parameter provides accurate predictions of the beneficial effect of fretting wear on fatigue life for pressure armour material [23].

A framework for fretting analysis of the pressure armour wire of flexible marine risers has been previously outlined [25]. This design framework is further developed here and a design study for a flexible riser jumper is conducted. Figure 1 shows the design framework implemented in this paper for service life prediction of the nub-groove contact in the pressure armour layer of a flexible riser jumper. A global-local computational methodology, with inputs from an experimental test regime, for analysis of the pressure armour layer in flexible risers is presented. The methodology consists of a hierarchy of models, including a global riser dynamic model, geometrical and analytical riser sub-models, and an axisymmetric nub-groove local contact model. The contact conditions, i.e. coefficient of friction and wear coefficient, in the local model are identified experimentally. Fretting testing of pressure armour material under representative loading conditions is conducted using a recently developed fretting rig. The representative loading conditions are determined from a combination of global riser models and geometrical and analytical riser sub-models, as depicted in Figure 1. Experimental testing of the pressure armour material under representative loading conditions allows for quantification of key fretting variables, including coefficient of friction and wear rate, which, in turn, control contact pressure evolution, relative slip and sub-surface stresses in this complex geometry.

Running condition fretting maps are developed for the riser jumpers, representing the key functional relationships between global riser variables and the local nub-groove fretting variables. These maps facilitate identification of the critical riser curvatures for minimum predicted numbers of cycles to crack initiation, applied here for a range of riser design geometries in the design study. The methodology provides crack initiation and propagation predictions for the range of riser geometries using the three-dimensional critical-plane Smith-Watson-Topper (SWT) multiaxial fatigue indicator parameter and a weight function method, respectively. Furthermore, the work presented here can be adapted to develop a riser design tool using a stand-alone computationally-efficient analytical fretting wear-fatigue model [26], based on the work of Hills et al. [27] and Nowell [28].

# Methodology

Figure 2 shows the framework for the methodology implemented in this paper. Global and local analyses are combined to predict the fretting fatigue life of the pressure armour wire nub-groove contact. The global analyses are conducted using the finite element software code Flexcom [29–34], to predict dynamic riser response, such as time histories of axial tension, F(t), and riser curvature, κ(t), in response to dynamically-imposed vessel motion and sea-state loading conditions. Geometrical and analytical models are used to identify the local FE (axisymmetric) riser model loading conditions, specifically applied displacement, Δapp(t), additional pressure pa(t), based on global riser curvature, κ(t), and axial tension, F(t), respectively, as shown in the flowchart of Figure 2. Local nub-groove fretting contact is analysed using an FE axisymmetric model, as described previously by the authors [22]. Fretting fatigue life is predicted from the resulting contact stresses, slips and surface traction results using the Dfret-SWT parameter, also previously presented by the authors [23]. Hence, wear is not explicitly modelled, thus reducing the computational time for local fretting analysis. Crack propagation is estimated using a weight function approach and hence, total life is obtained for the fretting regions of the risers.

# Global Riser Analysis

## General

This study focuses on a single line hybrid riser (SLHR) with a flexible riser jumper connecting to a floating production, storage and offloading vessel (FPSO) as shown schematically in Figure 3; three different dynamic flexible riser jumper geometries (internal diameters) are analysed (see Table 1). The water depth from the seabed to MSWL (mean sea water level, as shown in Figure 3) is 2030 m. This is close to the boundary between what is considered to be deep and ultra-deep (> 2,133 m) drilling. As oil reserves in shallower waters deplete, the oil and gas industry are pushed to extract oil at depths once considered too deep for hydrocarbon exploration and extraction. The configuration of SLHR with riser jumpers is often used to extract hydrocarbons at ultra-deep seas. The flexible riser jumpers fully decouple the SLHR from vessel motions; the vessel motions, in turn, are decoupled from the wellhead at the seabed, making hydrocarbon extraction safer.

Cyclic loading of dynamic flexible risers due to wave and current loading induce axial tension and curvature variations along the risers, leading to micro-scale motion between nub-groove contacts in the pressure armour wires. The full study of the behaviour of the flexible dynamic jumpers requires the consideration of long-term cyclic loading.

## Riser configuration and geometrical properties

The FPSO and SLHR hang-off points are typically critical sections for fatigue along the flexible riser due to the high tensions and curvature variation. The sag bend location can also be critical for fatigue, due to the high curvature variation in this area. However, the full length of the risers is analysed here since critical fretting conditions (PS) may not necessarily coincide with the locations of maximum loading or displacement.

For the three flexible riser jumper geometries (11 inch, 9 inch and 6 inch) investigated here, the inner diameter is based on the required capacity and internal pressure of the riser. The external diameter is used for calculation of drag and buoyancy of the riser. The internal diameter and pipe mass are used to calculate the buoyancy contribution of the internal fluid; the pipe mass also influences the resultant axial force. The internal fluid properties (density and internal pressure) for each riser jumper are also provided in Table 1. These fluid densities and pressures correspond to fluids that are typical of offshore oil and gas extraction, such as water and gas injection. Other geometric properties used for the global riser analysis are provided in Table 2. The external (hydrostatic) pressure, which is dependent on water-depth, and therefore, location along the riser, is zero at the FPSO hang-off; at the SLHR hang-off point, external hydrostatic pressure is 2 MPa.

The submerged sections of the risers experience hydrodynamic forces due to the environmental loading [35]. Generally, drag forces dominate for extreme waves and inertia forces are dominant for smaller waves. Drag and inertia coefficients of 1.2 and 1.8, respectively, are included in the global analysis for all riser geometries.

## Vessel and environmental loading

The vessel motions due to the wave action are modelled using vessel response amplitude operators (RAOs). The motion of all points on the vessel are calculated using rigid body dynamics from a reference point (typically vessel center of gravity) and the associated RAOs. A vessel has six degrees of freedom, three translational and three rotational, as shown in Figure 3. The RAO data for all six degrees of freedom used for this study were obtained from a database of typical FPSO data for this type of riser configuration and sea conditions (courtesy of Wood plc.).

A piecewise linear current profile is used to model the horizontal current velocity distribution that can vary in magnitude and direction with depth. For this study, the direction of the current is assumed to be constant, 0⁰ from the y-axis (see Figure 3).

The riser system is subjected to six sea state (regular Airy wave) load cases defined by wave height and period (as shown in Table 3). These wave heights and periods are representative of extreme dynamic loading on the flexible riser jumper from swell and sea wave conditions (Wood plc. met-ocean data). A riser can experience up to 106 cycles of each load-case in a year of operation. All wave directions are the same, except for load case 1, where the wave direction is at 90⁰ with respect to the other load cases. This allows for the effect of wave direction to be considered, by comparing load cases 1 and 3.

# Global-local conditions

## General

This section describes analytical and geometrical models employed to establish global-local relationships for (i) global axial force to local nub-groove contact pressure and (ii) global riser curvature to local nub-groove slip, as shown in the flowchart of Figure 2.

To the authors knowledge, there are no existing methods to identify nub-groove fretting conditions, in particular, contact slip, based on global riser dynamic analysis results. The work of Burke and Witz [18] identified fretting as a damage mechanism in the nub-groove contact region. This work considered the non-conforming helical profile of the nub-groove contact to be similar to a cylinder-on-flat contact. However, the nub-groove contact loading conditions were not quantified. As discussed earlier, Féret and Bournazel [20] developed theoretical approach for risers under axisymmetric loading to calculate contact stresses and slips in the tensile armour layer. Since the pressure armour layer nub-groove articulations are driven by global riser curvatures rather than axisymmetric loading conditions, the theoretical approach used by Féret and Bournazel [20] cannot be used for pressure armour layer nub-groove contacts. The aim of this section is to quantify the global-local response and, hence, provide data for the formulation of material response fretting maps (MRFM) and running conditions fretting maps (RCFM).

## Global riser axial tension

A key design criterion for flexible risers is the avoidance of compressive buckling. Each layer in a flexible riser has a specific function. The function of the tensile armour (wire) layers is to support the axial tensile forces experienced by the riser. The lay angle of the helical tensile armour layer is typically between ±30 and ±40⁰ to the longitudinal axis of the riser; therefore, tensile forces on this layer cause the wires to straighten slightly. This straightening causes pressure on layers under the tensile layer, such as the pressure armour layer. Therefore, global axial tensile force causes additional external pressure on the pressure armour layer, pa, which can be expressed as follows [36]:

|  | (1) |
| --- | --- |

where m is the number of tensile armour wires in the tensile armour layer, Fw is the axial force in each tensile armour wire, Rw is the radius at which the tensile armour layer lies with a lay angle of α and 2πRpa is the circumference of the pressure armour layer. This analytical solution has been validated and used extensively, e.g. [37,38].

## Global riser curvatures

In this section, a geometrical model is presented to relate global riser curvature-applied displacement for a local FE model of the riser nub-groove contact, as outlined in Figure 2. In order to develop this model, some assumptions are made, as follows:

- Plane sections remain plane

- The outer pressure armour layer slides relative to the inner layer under bending

- No ovalisation of the riser occurs under bending

- The angle of rotation, θ, remains small (Rθ ≈ l, see Figure 4)

- The length of riser modelled, l, is small with respect to the bend radius, R

- Pure bending occurs about the centre line of the riser

- Only elastic behaviour occurs (i.e. plasticity is neglected)

- No additional nub-groove contact pressure occurs due to riser bending

The length of the riser section modelled, l, in the simplified axisymmetric model is 15 mm, as described below, and the minimum bend radius of a riser is typically 1.5 to 3 m (this is the storage radius of the pipe - the bend radius during operation is much larger). Therefore, the angle of rotation, θ, remains small (maximum 0.3⁰ to 0.6⁰), and the section of the riser modelled is small compared to the bend radius (~ 1 %). A schematic of the geometrical model used to develop the riser curvature-applied displacement, κ-Δapp, relationship is shown in Figure 4, where curvature κ, is given by the inverse of bend radius, R.

From the geometrical construction (model) of Figure 4, the angle of rotation, θ, can be expressed in terms of riser bend radius, R, and length of riser section modelled, l, as follows:

| θ≈2sin-1κl2 | (2) |
| --- | --- |

where l is as shown in Figure 4. Assuming that the outer pressure armour layer slides as a rigid body relative to the inner layer, the relative applied displacement, Δapp, can be approximated as follows, for small θ:

| Δapp≈±yθ | (3) |
| --- | --- |

where y is the distance from the central riser axis to the nub-groove interface, as shown in Figure 4.

# Local nub-groove analysis

## Experimental characterisation of fretting of pressure armour material

A custom fretting test rig was developed and designed to measure coefficient of friction and quantify fretting behaviour of materials, in particular, pressure armour wire material. The primary function of these experimental tests is to validate the previously published use of a representative pressure armour material for the characterisation of coefficient of friction and wear coefficient [21]. The results from the previous study were used to develop computational wear models for the nub-groove contact [23]. The development of this test rig has allowed for validation of these previously-developed computational wear models for pressure armour material and under more representative local loading conditions, i.e. maximum contact pressure and contact slip, which have been calculated from the global-local riser analysis described here.

Figure 5 shows the labelled fretting rig. The main components include crossed-cylinder specimen configuration, a linear drive line with a piezoelectric actuator to produce reciprocating displacement and a collet to fix the test specimen in place, dead weight normal loading and displacement sensor (DVRT) and force sensors connected to a PC via a data acquisition unit.

The use of a piezoelectric actuator allows for closed-loop controlled cyclical displacements of ± 1 to 15 μm and a maximum tangential force of 172 N (resulting in typical of riser nub-groove contact pressures of 700 to 1700 MPa). A piezoelectric force sensor and DVRT are used to measure the tangential force and displacement, respectively. A LabView program has been developed to allow for simultaneous actuator control and data acquisition to take place.

Fretting experiments were conducted on pre-drawn pressure armour, pearlitic steel material [39] using the newly-designed fretting test rig [26]. This material has the same chemical composition as the riser material; however, the microstructure is different to that of the cold formed pressure armour wire profile. A crossed cylinder specimen configuration was used, as shown in Figure 5. This configuration allows high contact pressure tests to be conducted, representative of nub-groove contact pressures. The specimens were ground to a roughness (Ra) of 0.4 to 0.7 m. The test methodology employed was based on ASTM standard test methods [40,41].

A dead-weight configuration was used to provide a constant normal load, P, throughout the test. The displacement, Δ*, throughout the test is also measured at a location as close as possible to the contact using a DVRT. Similarly, the tangential force is measured using a piezoelectric force sensor. The load and displacement signals are sampled at a rate of 250 per fretting cycle for all the experiments (i.e. 500 Hz).

Real-time visualisation of sensor outputs is used to continuously monitor the fretting contact behaviour. This data is processed to give fretting loop evolution for each test; idealised gross-slip and partial-slip fretting loops are shown in Figure 6. The measured displacement amplitude (at the DVRT), Δ*, is not the same as the contact slip due to elastic deformation of the specimen and rig compliance. Since the fretting contact region is separated from the location of the displacement sensor, the measured displacement amplitude, is always slightly larger than the contact slip amplitude, δ*; this is due to elastic deformation of the specimen and rig compliance. The contact slip amplitude (δ*) can be derived by measuring the displacement at zero force, as illustrated in Figure 6.

Traditionally, the coefficient of friction (CoF) used by researchers was based on the Coulomb-Amontons’ friction law. This method is applicable for an idealised experimental fretting loop, as indicated in Figure 6(b), with CoF given by:

|  | (4) |
| --- | --- |

where Q* is the measured tangential (frictional) force during sliding and P is the applied normal load.

Fouvry et al. [42] observed that the tangential force does not remain constant through the sliding part of the fretting loop, as illustrated in Figure 6(c). It was suggested [42] that the peaks at the end of each sliding section of the fretting loops are caused by ploughing, which occurs during fretting when material build-up occurs at the wear scar edges. To minimise the influence of the ploughing effect on calculation of CoF, Fouvry et al. [42] proposed that the energy dissipated per cycle could be used to define a coefficient of friction which is more representative of the overall behaviour of the contact. The energy coefficient of friction (ECoF) [42] is defined as follows:

|  | (5) |
| --- | --- |

where Ed is the dissipated energy per cycle (area of the fretting loop, see Figure 6 (c)) and  is the total sliding distance per cycle [42]. The maximum contact slip, δ*, can be derived by measuring the displacement at zero force, as illustrated in Figure 6. For partial slip, tractional coefficient is used, which is given by ECoF for a PS fretting loop shown in  Figure 6(a).

Experiments were conducted for a range of normal loads, P. A summary of the fretting test parameters is provided in Table 4. Tests were conducted at ambient temperature (typically ~20 ⁰C), at a frequency of 2 Hz for a duration of 300,000 cycles. Using Young’s modulus of 189 GPa and a Poisson’s ratio of 0.33 for this steel, Hertzian contact mechanics [43] for a crossed-cylinder contact was used to determine the initial contact semi-width, a, and initial maximum contact pressure, po. The values of a and po are also presented in Table 4. Contact semi-width ranges from 80 μm to 157 μm and maximum contact pressure ranges from 890 MPa to 1730 MPa.

Figure 7 presents the measured evolution of the traction coefficient with number of fretting cycles for crossed-cylinders under different normal loads. The development of ECoF for gross slip fretting is presented in Figure 7(a); the stabilised ECOF values range from 0.59 to 0.65 in GS. Higher normal loads give lower ECoF values, resulting in partial slip conditions. The development of traction coefficient for partial slip fretting is presented in Figure 7(b). The traction coefficient for a normal load of 43 N (contact pressure ~ 1350 MPa) shows a step down from about 0.52 to about 0.42 at about 103 cycles.

The wear volume, V, was calculated as the volume of material removed from the reference surface, as described in [44]. This provided data to calculate the Archard wear coefficient using the equation:

| k=V4NPδ* | (6) |
| --- | --- |

where V is the wear volume, 4Nδ* is the sliding distance over N cycles, k is Archard wear coefficient and P is the normal load applied to the specimen. An average wear coefficient was calculated as 1.09 × 10-8 MPa-1, with a standard deviation of 1.4 × 10-9 MPa-1, from the gross slip experiments conducted here, with varying P and δ*. This is the same Archard wear coefficient as previously calculated from cylinder-on-flat experiments using representative pressure armour material that was subsequently used for the development of computational wear models [23].

## Finite element analysis of nub-groove contact conditions

Following global-local riser geometric analysis, combined with experimental nub-groove fretting characterisation, the local nub-groove slip, δ, is identified as a function of applied riser displacement, Δapp, from the axisymmetric riser model of [22]. The pressure armour wire of a flexible pipe is helically wound at an angle of approximately 95° to the longitudinal direction of the pipe, i.e. approximately circumferential to the pipe. Hence, it was assumed [22] that the helix angle of the pressure armour wire can be ignored for the purpose of these simulations, so that the riser can be modelled as axisymmetric. Furthermore, a repeated axial segment of the riser pressure armour wire was modelled to reduce computational time; this is described in detail in [22]. The key features of this model are the contact conditions for fretting stick-slip conditions. This is achieved through a combination of the master-slave algorithm with finite sliding contact within Abaqus and Coulomb friction based on the Lagrange multiplier contact algorithm. Further model details are outlined in [22].

In this study, the internal and external pressure for the riser are outlined in Table 1; the additional pressure due to the straightening of the tensile armour wires, pa, given by Equation (1), is also modelled. The cyclic applied displacement, ±Δapp, is identified by substituting Δκ for κ in Equations (4) and (5), where the global curvature range, Δκ = κmax - κmin, where κmax and κmin are the maximum and minimum curvatures that occur at a specific point on the riser throughout the dynamic simulation. The riser curvature histories in this study are typically symmetrical about the neutral axis of the riser (see Results section and Figure 8).

## Crack initiation

The numbers of cycles to crack initiation (10 μm crack) have been calculated using the methodology outlined in [23]. This implements the fretting damage parameter proposed by Ding et al. [24], Dfret, along with critical-plane SWT fatigue indicator parameter for the nub-groove fretting contact in flexible marine risers as outlined in Figure 2. Therefore, wear is not explicitly modelled here. The Dfret method introduces a surface damage factor for the effects of fretting wear in conjunction with critical-plane SWT approach, developed to incorporate the effects of slip and surface wear damage on crack initiation, as follows:

|  | (7) |
| --- | --- |

where

|  | (8) |
| --- | --- |

where C, m, and  are material parameters; τ and δ are the local contact shear and slip, respectively. (τδ)th has been estimated here as a value slightly greater than the value at the partial slip-gross slip transition, based on FE simulations and following Ding et al. [24]. For τδ < (τδ)th, the unmodified SWT value is used (i.e. Dfret = 1); for τδ > (τδ)th, the SWT-Dfret value is used [22]. The low- and high-cycle fatigue parameters and Dfret material parameters used here are presented in Table 5. This model was developed using wear coefficients calculated from experiments on representative pressure armour material but remains valid based on the experimental results of the present work on actual pressure armour material, as presented above in Section 5.1.

## Crack propagation

This section describes the estimation of crack propagation using a weight-function approach [45], as outlined in Figure 2. The total fretting fatigue life can then be calculated as the sum of crack initiation and propagation lives. The weight function approach was used for the PS regime, where it is assumed that wear rate is low and therefore, wear has little or no effect on substrate stresses. Since wear is not explicitly modelled, the contact stress gradient in the nub-groove contact region is expected to reduce up to a depth where constant stress is observed (far from the nub-groove contact surface). In an attempt to capture the effect of fretting stress relaxation due to wear, stress reduction factors have been estimated based previously presented stress relaxation trends perpendicular to the contact surface due to wear  [26]. These estimated stress gradients have been used here to estimate crack propagation in the gross-slip regime.

The weight function method used for PS is described below. In fretting experiments, cracks have been observed to grow at a shallow angle initially, typically between 5⁰ and 20⁰ [46], being influenced by the contact load. The crack will grow in this manner for up to three microstructural grain sizes [47]. Next the direction of the crack propagation changes to approximately perpendicular to the contact surface; here the propagation direction is influenced by the bulk stress in the pressure armour wire [26].

Due to stress gradients close to the contact surface in fretting, crack propagation is a complex issue [48]. Houghton et al. [45] used weight functions based on the work of Nicholas et al. [49] to analyse the mixed-mode cracking of Ti-6Al-4V to predict multiaxial fretting fatigue. Crack location was found from SWT critical plane calculations, and local stress ranges were identified for Mode I and Mode II crack propagation. The weight functions, hI and hII, for Mode I and Mode II crack growth, respectively, are defined as [50]:

|  | (9) |
| --- | --- |
|  | (10) |

where ρ = x/a and α = a/W, a is the crack length, x is the distance along the crack, W is the specimen width and Aυ,μ is the influence coefficient for each weight coefficient [50]. Mode I and Mode II stress intensity factors are defined as:

|  | (11) |
| --- | --- |
|  | (12) |

where Δσxx and Δσxy are the local FE-predicted normal and shear stresses during crack growth in Mode I and Mode II, respectively. The effective stress intensity factor is:

|  | (13) |
| --- | --- |

where:

|  | (14) |
| --- | --- |

where R is the stress ratio > 0 and n is a material constant, where 0 < n < 1. In this work, n was chosen to be 0.5. The incremental crack growth, Δa, is defined by:

|  | (15) |
| --- | --- |

where ΔN is a cycle jumping factor. The process crack growth calculation is incrementally repeated until the crack length, a, propagates to a length of 2 mm; at this distance away from the contact region, the primary stress is in the axial direction. Previously published Paris coefficients for a dual phase pearlite-ferrite cold-formed steel were used here [51]. This methodology has been successfully validated against results obtained from the Paris equation for a plane fatigue case. This method includes the El-Haddad correction for short crack growth [52].

# Results

## Global analysis

Key output variables from the global analysis are (i) time histories of riser curvature and axial tension and (ii) envelopes of the minimum and maximum riser curvatures and axial tensions along the length of the riser. This data allows for detailed design of flexible risers based on dynamic responses to various sea state loadings.

Normalised time histories of resultant rise curvature (κ/κmax), axial tension (F/Fmax) and internal and external pressures (p/pmax) results from dynamic riser analysis are shown in Figure 8; these results are for the SLHR hang-off point. The internal and external pressure remain constant with respect to time. In all load-cases, riser curvature is seen to be 180⁰ out of phase with axial tension. The axial force is tensile at all times.

The minimum and maximum axial force and riser curvature along the length of the 6 inch riser under load case 1 are presented in Figure 9. Note that l in these plots is the length of the riser, measured as curvilinear distance along the riser from the FPSO hang-off point (see Figure 3). The end-point (l = 550 m) corresponds to the SLHR hang-off point. Clearly, the predicted location of maximum axial tension and riser curvature values is at the SLHR hang-off point. The SLHR has a more dynamic response to the sea-state loading than the FPSO, therefore, the SLHR hang-off point is the most dynamically loaded point on the flexible riser. The results from the other load-cases analysed for the three risers are consistent with these.

## Global-local loading conditions

A typical predicted relationship between nub-groove contact displacement (δ) and riser curvature range (Δκ) from FE analysis is shown in Figure 10. This response is obtained from the global riser curvature results, Equations (4) and (5) and the local FE riser model [22]. The curvature-slip response shows two types of trends, depending on slip regime, for finite sliding friction; a quadratic trend is observed for PS and a linear trend is observed for GS. To achieve slip, i.e. overcome friction, a certain threshold curvature range, Δκth, is required; above this threshold value, curvature-slip response is much “softer”, i.e. a smaller change in curvature results in a larger change in slip. The value of Δκth depends on riser geometry and loading conditions.

This combined geometrical-axisymmetric FE method, as described in Figure 2, has been compared to the 3D riser model presented in [26]. This model, however, by necessity, uses a much coarser local contact mesh. The general trend is seen to be the same. Differences are attributed to the simplifying assumption of axisymmetry and the coarser mesh for the 3D riser model. The analysis of curvature-slip relationship requires further computational investigation and full-scale experimental validation, if possible.

Figure 10 shows the curvature-slip relationship for the different size risers. The effect of decreasing riser diameter is the same as that of increasing internal pressure and increasing CoF.

Threshold values of curvature range, Δκth, have been identified as the curvature range at which nub-groove contact transitions from PS to GS.

The Δκ-δ relationships shown above can be described by the following equations, for PS and GS:

|  | (PS) | (16) |
| --- | --- | --- |
|  | (GS) | (17) |

where A, B, and C for each riser geometry and nub groove sliding regime are given in Table 6. The values of Δκth are also provided in Table 6. Partial slip occurs at Δκ values less than Δκth.

The predictive methodology has been applied to the various dynamic load cases for each riser diameter. The results for maximum predicted slip for each riser are shown in Table 7. For the minimum loading conditions along the riser lengths, stick is predicted, whereas for the maximum conditions along the riser lengths, gross slip is typically predicted. Therefore, along the riser length, stick, partial slip and gross slip conditions are experienced for most load cases. Load case 5 results in maximum nub-groove contact displacements closest to PS-GS transition. Typically, the larger wave heights (see Table 3) result in larger nub-groove slips, i.e. load cases 1 to 3; this is due to larger vessel and SLHR motions associated with wave loading, and hence, larger global riser dynamic motions occur compared to load cases with lower wave heights, i.e. load cases 4 to 6.

## Predicted life to crack initiation

Figure 11 presents the predicted number of cycles to crack initiation versus nub-groove displacement for all three risers. At lower contact slips (< 2 μm) the 11 inch riser is most critical (lower predicted life); however, as contact slip increases (> 4 μm), the 6 inch riser is more critical in terms of predicted number of cycles to crack initiation.

Figure 12 presents the effect of riser curvature range, Δκ, on predicted number of cycles to crack initiation for all three risers. The difference between the riser configurations can be clearly seen. The PS regime occurs at much lower curvatures for the larger diameter risers (11 inch and 9 inch) than for the smaller diameter riser (6 inch). This is due to the different curvature-slip relationships for each riser, as shown in Figure 10. Figure 12 is an example of a RCFM; this can be used to identify the number of cycles to crack initiation from global riser curvature response.

Figure 12 also shows critical curvature range (Δκcrit) that relate to lower predicted lives (Ni < 5 × 104) for each riser. The smaller diameter (6 inch) riser has a large ranges of Δκcrit than the larger diameter risers (9 inch and 11 inch). These Δκcrit values can be used in conjunction with global riser analysis results (from global riser analysis) to identify locations of the riser where fretting fatigue cracking is likely to occur. For all load cases, Δκcrit occurs closest to the SLHR hang-off point (between 0.1 m and 5 m from hang-off point) for each of the risers. It must be noted that these values for Δκcrit are specific to each of these risers under the loading conditions outlined above and with a nub-groove CoF of 0.7.

## Predicted crack propagation

Figure 13 presents the predicted number of cycles to crack initiation and propagation against nub-groove displacement for the 6 inch riser. Estimated propagation life is much longer than predicted initiation life (typically by two orders of magnitude). This is due to the fact that the high fretting stresses are highly localised close to the nub-groove contact surface [22], and therefore, have little influence as the crack propagates. Figure 13 presents the effect of riser curvature range, Δκ, on predicted number of cycles to crack initiation and propagation for the 6 inch riser. Figure 14 presents the effect of riser curvature range, Δκ, on predicted number of cycles to crack initiation and propagation for all risers. This is an example of a RCFM and can be used to identify the number of cycles to failure from global riser dynamic response.

The predicted number of cycles to crack initiation for each load case and riser is presented in Figure 15 (a). The 11 inch riser is typically the most critical riser, in terms of predicted number of cycles to crack initiation, compared to the smaller diameter risers (9 inch and 6 inch). The load case with the most difference between predicted riser lives is load case 5; this corresponds to the load case with the lowest wave height and period. Comparing the predicted number of cycles to crack initiation for load cases 1 and 3 indicates that wave direction has an effect on predicted life; therefore, in a full riser analysis, a full range of wave loading directions should be considered. The location where Δκcrit occurs along the riser length is short relative to the length of the riser, typically ≤ 2 m compared to overall riser length of 550 m. This is approximately 0.4 % of the riser length that is critical for fretting design.

Figure 15 (b) presents the predicted number of cycles to failure for all three risers and all load cases. There is a significant increase in life for all risers and load cases compared to predicted initiation life (shown in Figure 15 (a)). There is less scatter between the results for failure than for initiation, indicating that crack propagation is dominant over crack initiation, due to stresses perpendicular to the surface decreasing rapidly with increasing depth into the substrate, resulting in similar predicted propagation lives for all risers and load cases.

Currently, there is no standard procedure for the analysis of fretting in the pressure armour layer. Typically, risers are analysed for fatigue behaviour of the tensile armour layer. For this study, the fatigue life of tensile armour layer has also been predicted using Layercom, a tensile armour layer analytical fatigue predictive tool (Wood plc., Galway, Ireland). The analytically predicated lives for the tensile armour layer ranged from 5.9 × 105 to 1.3 × 107. This is a similar range to the total fretting fatigue life predicted for the pressure armour layer in this study (see Figure 15 (b)). The load cases investigated here are extreme loading scenarios; by modelling stochastic sea states, less conservative and more realistic life predictions can be calculated using the same predictive methodologies for both fatigue of the tensile armour layer and fretting fatigue of the pressure armour layer.

# Conclusions

A global-local computational methodology is presented for fretting fatigue of the pressure armour layer in flexible risers under realistic loading conditions. The methodology consists of a hierarchy of models, including a global riser dynamics model, geometrical and analytical riser sub-models, and an axisymmetric nub-groove local contact model. A new fretting test rig is also presented for representative testing of flexible riser pressure armour contact conditions. The key functional relationships between global riser variables (running conditions) and local nub-groove fretting variables have been developed to facilitate identification of critical riser curvatures for fatigue crack initiation and total life in different riser design geometries. Running condition fretting maps are thus presented for the different riser geometries. The resulting predicted fretting fatigue lives are found to be in the same range as tensile armour layer plain fatigue lives.

The riser design study facilitated new insight into fretting in risers, such as:

- The critical curvature range for partial slip (and hence minimum fretting life) reduces with increasing riser diameter

- Propagation life is predicted to be significantly (two orders of magnitude) longer than initiation life, due to the significant gradient in stress with increasing depth into the armour wire substrate

This study has highlighted the requirement for further investigation into the effect of global riser curvature on local nub-groove contact slip. To quantify this effect, 3D riser modelling including detailed geometry of each layer and contact pressures and slip between layers is required. These 3D models should be validated using full-scale riser testing; this is a difficult task since contact slip is difficult to measure experimentally, especially in the small space within the pressure armour layer. However, it may be feasible to implement displacement or position sensors to measure the effect of riser curvature on pressure armour wire displacement; FE modelling can then be used to predict local nub-groove contact slip. The present work will inform new flexible riser design codes and standards which may allow for reduced safety factors for fatigue design of risers [53].

# Acknowledgements

The authors would like to thank the Irish Research Council and Wood plc. for funding of this project through the Enterprise Partnership Scheme (EPSPG/2013/638), the National University of Ireland for funding through an NUI Travelling Scholarship. The authors also wish to acknowledge the support of the NUI Galway Engineering Building technical staff; particularly Mr. Bonaventure Kennedy and Mr. Patrick Kelly.

# References

[1]	Dobromirski J. Variables of fretting process: Are there 50 of them? Stand Frett Fatigue Test Methods Equip 1992:60-60–7. doi:10.1520/STP25816S.

[2]	Glaeser WA. Tribology, the science of combating wear. Battelle, Columbus Division; 1981.

[3]	Ding J, McColl IR, Leen SB, Shipway PH. A finite element based approach to simulating the effects of debris on fretting wear. Wear 2007;263:481–91. doi:10.1016/j.wear.2006.12.056.

[4]	Wavish PM, Houghton D, Ding J, Leen SB, Williams EJ, McColl IR. A multiaxial fretting fatigue test for spline coupling contact. Fatigue Fract Eng Mater Struct 2009;32:325–45. doi:10.1111/j.1460-2695.2009.01334.x.

[5]	Ruiz C, Boddington PHB, Chen KC. An investigation of fatigue and fretting in a dovetail joint. Exp Mech 1984;24:208–17.

[6]	Nowell D, Dini D, Hills DA. Recent developments in the understanding of fretting fatigue. Eng Fract Mech 2006;73:207–22. doi:10.1016/j.engfracmech.2005.01.013.

[7]	Cruzado  a., Hartelt M, Wäsche R, Urchegui M a., Gómez X. Fretting wear of thin steel wires. Part 1: Influence of contact pressure. Wear 2010;268:1409–16. doi:10.1016/j.wear.2010.02.017.

[8]	Mulvihill DM, Kartal ME, Olver AV, Nowell D, Hills DA. Investigation of non-Coulomb friction behaviour in reciprocating sliding. Wear 2011;271:802–16. doi:10.1016/j.wear.2011.03.014.

[9]	Kartal ME, Barber JR, Hills D a., Nowell D. Partial slip problem for two semi-infinite strips in contact. Int J Eng Sci 2011;49:203–11. doi:10.1016/j.ijengsci.2010.11.007.

[10]	Mugadu A, Hills DA, Nowell D. Modifications to a fretting-fatigue testing apparatus based upon an analysis of contact stresses at complete and nearly complete contacts. Wear 2002;252:475–83. doi:10.1016/S0043-1648(02)00007-8.

[11]	Pearson SR, Shipway PH. Is the wear coefficient dependent upon slip amplitude in fretting? Vingsbo and Söderberg revisited. Wear 2015;330–331:93–102. doi:10.1016/j.wear.2014.11.005.

[12]	Fouvry S. Shakedown analysis and fretting wear response under gross slip condition. Wear 2001;251:1320–31. doi:10.1016/S0043-1648(01)00767-0.

[13]	Hills DA, Nowell D. Mechanics of Fretting Fatique. Springer Netherlands; 1994.

[14]	Araújo J a., Nowell D. The effect of rapidly varying contact stress fields on fretting fatigue. Int J Fatigue 2002;24:763–75. doi:10.1016/S0142-1123(01)00191-8.

[15]	Araújo JA, Nowell D, Vivacqua RC. The use of multiaxial fatigue models to predict fretting fatigue life of components subjected to different contact stress fields. Fatigue Fract Eng Mater Struct 2004;27:967–78. doi:10.1111/j.1460-2695.2004.00820.x.

[16]	Araújo JA, Nowell D. Analysis of pad size effects in fretting fatigue using short crack arrest methodologies. Int J Fatigue 1999;21:947–56. doi:10.1016/S0142-1123(99)00077-8.

[17]	Nowell D, Dini D. Stress gradient effects in fretting fatigue. Tribol Int 2003. doi:10.1016/S0301-679X(02)00134-2.

[18]	Burke RN, Witz JA. NFretting Fatigue of Flexible Pipe Pressure Armour. 3rd Eur. Conf. Flex. Pipes, Unbilicals Mar. Cables - Mater. Util. Cycl. Therm. Load., 1995.

[19]	Perera SDR, Fernando US, Sheldrake T, Clements R. An investigation into fretting behaviour in pressure armor wires of unbonded flexible pipes. Vol. 3 Pipeline Riser Technol. CFD VIV, vol. 3, San Diego, California, USA: 2007, p. 363–70. doi:10.1115/OMAE2007-29393.

[20]	Féret JJ, Bournazel CL. Calculation of stresses and slip in structural layers of unbonded flexible pipes. J Offshore Mech Arct Eng 1987;109:263. doi:10.1115/1.3257019.

[21]	O’Halloran SM, Harte AM, Shipway PH, Leen SB. An experimental study on the key fretting variables for flexible marine risers. Tribol Int 2018;117:141–51. doi:10.1016/J.TRIBOINT.2017.07.032.

[22]	O’Halloran SM, Harte AM, Connaire AD, Leen SB. Modelling of fretting in the pressure armour layer of flexible marine risers. Tribol Int 2016;100:306–16. doi:10.1016/j.triboint.2016.02.040.

[23]	O’Halloran SM, Shipway PH, Connaire AD, Leen SB, Harte AM. A combined wear-fatigue design methodology for fretting in the pressure armour layer of flexible marine risers. Tribol Int 2017;108:7–15. doi:10.1016/J.TRIBOINT.2016.10.020.

[24]	Ding J, Houghton D, Williams EJ, Leen SB. Simple parameters to predict effect of surface damage on fretting fatigue. Int J Fatigue 2011;33:332–42. doi:10.1016/j.ijfatigue.2010.09.008.

[25]	O’Halloran S, Harte A, Connaire A, Leen S. The Prediction of Fretting Fatigue in the Pressure Armours of Dynamic Flexible Pipes. Offshore Technol. Conf., Offshore Technology Conference; 2016. doi:10.4043/26985-MS.

[26]	O’Halloran SM. Experimental Characterisation , Computational Modelling and Design Tool Development for Fretting Fatigue and Wear in Flexible Marine Risers. Ph.D. Thesis, Mechanical Engineering, National University of Ireland Galway, 2017.

[27]	Hills DA, Sackfield A, Paynter RJH. Simulation of Fretting Wear in Halfplane Geometries: Part 1—The Solution for Long Term Wear. J Tribol 2009;131:31401. doi:10.1115/1.3118785.

[28]	Nowell D. Simulation of Fretting Wear in Half-Plane Geometries—Part II: Analysis of the Transient Wear Problem Using Quadratic Programming. J Tribol 2010;132:21402. doi:10.1115/1.4000733.

[29]	Lane M, O’Sullivan T, Grealish F, Kavanagh K, Thompson H. Advanced Frequency Domain Analysis Techniques for Steel Catenary Risers. Offshore Technol. Conf., Houston, Texas, USA: 2001, p. 1–14. doi:10.4043/13017-MS.

[30]	Lang DW, Connolly A, Lane M, Connaire AD. Advances in Frequency and Time Domain Coupled Analysis for Floating Production and Offloading Systems. Int. Conf. Offshore Mech. Arct. Eng., Halkidiki, Greese: 2005. doi:10.1115/OMAE2005-67396.

[31]	O’Brien PJ, Lane M, McNamara JF. Improvements to the convected co-ordinates method for predicting large deflection extreme riser response. Int. Conf. Offshore Mech. Arct. Eng., Oslo, Norway: 2002. doi:10.1115/OMAE2002-28237.

[32]	O’Brien PJ, McNamara JF, Lane. Three dimensional finite displacements and rotations of flexible beams including non-equal bending stiffnesses. Int. Conf. Offshore Mech. Arct. Eng., Cancun, Mexico: 2003. doi:10.1115/OMAE2003-37372.

[33]	Smith R, Carr T, Lane M. Computational tool for the dynamic analysis of flexible risers incorporating benging hysteresis. Int. Conf. Offshore Mech. Arct. Eng., San Diego, California, USA: 2007. doi:10.1115/OMAE2007-29276.

[34]	Connaire A, O’Brien P, Harte AM, O’Connor A. Advancements in subsea riser analysis using quasi-rotations and the Newton-Raphson method. Int J Non-Linear Mech 2015;70:47–62. doi:10.1016/j.ijnonlinmec.2014.10.021.

[35]	Flexcom Introductory Manual, April 2014: Wood Group Kenny. n.d.

[36]	Lanteigne J. Theoretical Estimation of the Response of Helically Armored Cables to Tension, Torsion, and Bending. J Appl Mech 1985;52:423. doi:10.1115/1.3169064.

[37]	McNamara JF, Harte AM. Three-dimensional analytical simulation of flexible pipe wall structure. J Offshore Mech Arct Eng 1992;114:69–75. doi:10.1115/1.2919961.

[38]	Hoffman J, Dupont W, Reynolds B. A fatigue-life prediction model for metallic tube umbilicals. Offshore Technol. Conf., 2001. doi:10.4043/13203-MS.

[39]	Ashton P. Micromechanical modelling of size effects in crack initiation with application to fretting fatigue and cold dwell fatigue. Ph.D. Thesis, Mechanical Engineering, National University of Ireland Galway, 2018.

[40]	ASTM International. ASTM D4170-10 Standard test method for fretting wear protection by lubricating greases 2010. doi:10.1520/D4170-10.

[41]	ASTM International. ASTM D7594-11 Standard test method for determining fretting wear resistance of lubricating greases under high Hertzian contact pressure using a high-frequency, linear-oscillation (SVR) test machine 2011. doi:10.1520/D7594-11.

[42]	Fouvry S, Duó P, Perruchaut P. A quantitative approach of Ti-6Al-4V fretting damage: Friction, wear and crack nucleation. Wear 2004;257:916–29. doi:10.1016/j.wear.2004.05.011.

[43]	Hertz H. Über die Berührung fester elastischer Körper. J Fur Die Reine Und Angew Math 1882;1882:156–71. doi:10.1515/crll.1882.92.156.

[44]	Warmuth AR, Pearson SR, Shipway PH, Sun W. The effect of contact geometry on fretting wear rates and mechanisms for a high strengthsteel. Wear 2013;301:491–500. doi:10.1016/j.wear.2013.01.018.

[45]	Houghton D, Wavish PM, Williams EJ, Leen SB. Multiaxial fretting fatigue testing and prediction for splined couplings. Int J Fatigue 2009;31:1805–15. doi:10.1016/j.ijfatigue.2008.12.005.

[46]	Dubourg M, Lamacq V. Stage II crack propagation direction determination under fretting fatigue loading: a new approach in accordance with experimental observations. Frett. Fatigue Curr. Technol. Pract., West Conshohocken, PA: ASTM International; 2000, p. 436-436–15. doi:10.1520/STP14746S.

[47]	Khotsyanovskii AO. Life prediction of titanium and aluminum alloys under fretting fatigue conditions using various crack propagation criteria. Part 1. Experimental and calculation techniques. Strength Mater 2010;42:683–704. doi:10.1007/s11223-010-9256-7.

[48]	McCarthy OJ, McGarry JP, Leen SB. Microstructure-sensitive prediction and experimental validation of fretting fatigue. Wear 2013;305:100–14. doi:10.1016/j.wear.2013.05.012.

[49]	Nicholas T, Hutson A, John R, Olson S. A fracture mechanics methodology assessment for fretting fatigue. Int J Fatigue 2003;25:1069–77. doi:10.1016/S0142-1123(03)00115-4.

[50]	Fett T, Munz D. Stress intensity factors and weight functions. Computational Mechanics Publications, Southhampton; 1997.

[51]	Li S, Kang Y, Kuang S. Effects of microstructure on fatigue crack growth behavior in cold-rolled dual phase steels. Mater Sci Eng A 2014;612:153–61. doi:10.1016/j.msea.2014.06.002.

[52]	El Haddad MH, Topper TH, Smith KN. Prediction of non propagating cracks. Eng Fract Mech 1979;11:573–84. doi:10.1016/0013-7944(79)90081-X.

[53]	API. Recomended practices for flexible marine risers, 17J. 2009.

Figures

Figure 1. Design framework for service life prediction of the nub-groove contact in the pressure armour layer of a flexible riser jumper.

Figure 2. Framework for fretting life prediction in flexible marine risers.

Figure 3. Schematic of SLHR and single riser jumper configuration. and definition of vessel degrees of freedom for RAOs.

Figure 4. Schematic of effect of bending on riser section and the geometrical model used to relate riser curvature to local applied displacement.

Figure 5. Design of fretting rig, without dead weight normal loading showing the crossed cylinder test speciment configuration.

Figure 6. Schematic diagram of an ideal fretting loop in the (a) partial slip regime, and (b) gross slip regime, (c) fretting loop with high tractional force peak at the ends of gross slip.

Figure 7. Measured evolution of ECoF and traction coefficients with number of fretting cycles for specimen with normal load of P for (a) gross-slip (contact pressure between 890 and 1270 MPa), and (b) mixed and partial slip conditions (contact pressure between 1350 and 1730 MPa).

Figure 8. Time histories of normalised results from dynamic riser analysis.

Figure 9. Envelope of minimum and maximum (a) axial tensile force and (b) riser curvature along the length of the 6 inch riser jumper.

Figure 10. Curvature-slip relationship for the different risers used in this study.

Figure 11. Number of cycles to crack initiation against nub-groove displacement for all three risers.

Figure 12. Number of cycles to crack initiation against dynamic riser curvature for all three risers and definition of Δκcrit for each riser case.

Figure 13. Effect of (a) nub-groove contact slip and (b) global riser curvature on number of cycles to crack initiation and propagation for the 6 inch riser.

Figure 14. RCFM for predicted number of cycles to failure for dynamic curvature range for all three risers.

Figure 15. Predicted numbers of cycles to (a) to crack initiation and (b) failure for all load cases and all three risers.

Tables

Table 1. Riser geometrical and internal fluid data.

Table 2. Riser geometrical data.

Table 3. Definition of regular wave load-cases for design study.

Table 4. Summary of the fretting test parameters.

Table 5. Identified low- and high-cycle fatigue constants and Dfret constants for pressure armour wire [23].

Table 6. A, B, and C for each riser geometry and nub-groove sliding conditions

Table 7. Maximum nub-groove contact displacements for each riser and load case.

Tables

Table 1. Riser geometrical and internal fluid data.

| Riser | Inner diameter (mm) | Outer diameter (mm) | Internal Fluid | Internal fluid density (kg/m3) | Internal pressure (MPa) |
| --- | --- | --- | --- | --- | --- |
| 11 inch | 279.4 | 404.5 | Water Injection | 1100 | 27.3 |
| 9 inch | 222.3 | 336.9 | Gas Injection | 285 | 32.4 |
| 6 inch | 152.4 | 362.8 | Gas Lift | 267 | 27.5 |

Table 2. Riser geometrical data.

| Riser | Bending stiffness, EI (Nm2) | Torsional stiffness, GJ (Nm2) | Axial stiffness, EA (N) | Mass (kgf/m) |
| --- | --- | --- | --- | --- |
| 11 inch | 1.34 × 105 | 1.34 × 106 | 1.25 × 109 | 279 |
| 9 inch | 3.71 × 104 | 3.71 × 105 | 9.52 × 108 | 208 |
| 6 inch | 4.91 × 104 | 4.91 × 105 | 6.62 × 108 | 165 |

Table 3. Definition of regular wave load-cases for design study.

| Load case | Wave height (m) | Wave period (s) |
| --- | --- | --- |
| 1 | 2.78 | 11.5 |
| 2 | 2.36 | 10.54 |
| 3 | 2.78 | 11.65 |
| 4 | 1.74 | 6.94 |
| 5 | 0.93 | 6.94 |
| 6 | 1.74 | 14.81 |

Table 4. Summary of the fretting test parameters.

| Test temperature | Ambient (typically 20⁰C) |
| --- | --- |
| Cylinder radii, R | 6 mm |
| Displacement amplitudes, Δ* | 1 to 6 m |
| Test duration, N | 300,000 cycles |
| Normal load, P | 12 to 90 N |
| Maximum Hertzian pressure, po | 890 to 1730 MPa |
| Initial contact width, a | 80 to 157 μm |
| Oscillation frequency | 2 Hz |
| Lubricant | None |

Table 5. Identified low- and high-cycle fatigue constants and Dfret constants for pressure armour wire [23].

| Parameter |  | Value |
| --- | --- | --- |
| (τδ)th |  | 0.4 MPa mm |
| C |  | 0.05 MPa-1 mm-1 |
| m | -0.85 |  |
| σ’f | 1710 MPa |  |
| ε’f | 0.2355 |  |
| b | -0.12 |  |
| c | -0.5742 |  |

Table 6. A, B, and C for each riser geometry and nub-groove sliding conditions

| Riser | A | B | C | B | C | Δκth |
| --- | --- | --- | --- | --- | --- | --- |
|  | (PS) |  |  | (GS) |  |  |
| 6 inch | 3.9 × 103 | 7.2 | -4.3 × 10-3 | 1.1 × 103 | -9.7 | 8.9 × 10-3 |
| 9 inch | 1.0 × 103 | 11.4 | -2.2 × 10-3 | 1.7 × 103 | -8.9 | 5.5 × 10-3 |
| 11 inch | 1.8 × 104 | 18.7 | -0.6 × 10-3 | 1.9 × 101 | -0.6 × 10-3 | 3.7 × 10-3 |

Table 6. Maximum nub-groove contact displacements for each riser and load case.

| Load case | 6 inch riser | 9 inch riser | 11 inch riser |
| --- | --- | --- | --- |
|  | δmax (μm) | δmax (μm) | δmax (μm) |
| 1 | 22.8 | 83 | 192.1 |
| 2 | 45.4 | 65.1 | 192.1 |
| 3 | 78.1 | 81.9 | 188.7 |
| 4 | 40.5 | 44.6 | 101.8 |
| 5 | 2.1 | 3.3 | 4.4 |
| 6 | 29.2 | 27.2 | 87.8 |

Figures

| Global riser analysis | Cross-sectional riser analysis | Local nub-groove pressure armour layer analysis |
| --- | --- | --- |

Figure 1. Design framework for service life prediction of the nub-groove contact in the pressure armour layer of a flexible riser jumper.

Figure 2. Framework for fretting life prediction in flexible marine risers.

Figure 3. Schematic of SLHR and single riser jumper configuration and definition of vessel degrees of freedom for RAOs.

.

Figure 4. Schematic of effect of bending on riser section and the geometrical model used to relate riser curvature to local applied displacement.

Figure 5. Design of fretting rig, without dead weight normal loading showing the crossed cylinder test speciment configuration.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 6. Schematic diagram of an ideal fretting loop in the (a) partial slip regime, and (b) gross slip regime, (c) fretting loop with high tractional force peak at the ends of gross slip.

Figure 7. Measured evolution of ECoF and traction coefficients with number of fretting cycles for specimen with normal load of P for (a) gross-slip (contact pressure between 890 and 1270 MPa), and (b) mixed and partial slip conditions (contact pressure between 1350 and 1730 MPa).

Figure 8. Time histories of normalised results from dynamic riser analysis.

| (a) | (b) |
| --- | --- |

Figure 9. Envelope of minimum and maximum (a) axial tensile force and (b) riser curvature along the length of the 6 inch riser jumper.

Figure 10. Curvature-slip relationship for the different risers used in this study.

Figure 11. Number of cycles to crack initiation against nub-groove displacement for all three risers.

Figure 12. Number of cycles to crack initiation against dynamic riser curvature for all three risers and definition of Δκcrit for each riser case.

| (a) | (b) |
| --- | --- |

Figure 13. Effect of (a) nub-groove contact slip and (b) global riser curvature on number of cycles to crack initiation and propagation for the 6 inch riser.

Figure 14. RCFM for predicted number of cycles to failure for dynamic curvature range for all three risers.

| (a) | (b) |
| --- | --- |

Figure 15. Predicted numbers of cycles to (a) to crack initiation and (b) failure for all load cases and all three risers.